# 1. Test Execution and Setup

## 1.1 Test Description

Pulse ID: J050271

## 1.2 Test / Part Criteria

General Data

- Test Regulation		Test performed according to the Stellantis test procedure B32-3210

- Sled Pulse [Number] 	Biltir: 26002Y018E008

- Test Object                          Test applicable on K0 complete front seat

- Test Fixture                         K0 front seat crash fixture for metal validation (simulating podium as rigid)

K0 footrest

K0 B-pilar

- Type of Dummy		HIII 50% M

- Dummy Setup Information

| Parameter | Coord. | Target | Actual |
| --- | --- | --- | --- |
| H-Point [mm] | X | 1163,97 < Hx < 1189,97 | 1182.29 |
|  | Y | +/-380 | -416.65 |
|  | Z | 614,38 < Hz < 640,38 | 631.32 |
| Heel Point at the footrest [mm] | X | 566.5 | 490 |
|  | Y | N.A. | N/A |
|  | Z | 275 | 276.3 |
| Feet angle [°] at the footrest | / | 40.0 | 40.1 |
| Pelvis Angle [°] | / | 22.5 ± 2.5 | 24.10 |
| Head Angle [°] | / | 0 ± 2.5 around Y and X-axis | 1.55/0.70 |
| Head COG [mm] | X | To record X-backset for info | Not necessary for this test |
|  | Z | To record Z-backset for info | Not necessary for this test |

| - Belt System | 1 - Original belt buckle for K0 project NAFTA variant 2 - Original PLP for K0 project NAFTA variant 3 - Original belt for K0 project with 600daN NAFTA variant Belt elongation = (Input from FEA required) |
| --- | --- |

- Belt Coordinates D-Loop

D-loop fixing point:

X = 1480.7 mm

Y = + 721.1 mm

Z = 1276.3 mm

Retractor fixing point:

X = 1413.09 mm

Y = + 752.9 mm

Z = 388.7 mm

- Belt Coordinates Lower Outboard

PLP fixing point on Bpilar:

X = 1425.2 mm

Y = + 700.3 mm

Z = 251.7 mm

- Belt Force Sensor                              1 - Diagonal belt sensor => Distance to the D-ring = 200mm

2 -  Lap belt sensor outboard => Minimum distance to the outboard low fixation = 150mm

- Firing Time Pretensioner [ms]	PLP = 20 ms

The PLP device shall be blocked (integrated in the Bpilar design)

- Firing Time Retractor [ms]		Retractor = 10 ms

- Additional information		Pull in force = (input to be filled by FEA)

pretensioner 			Pull in displacement = (input to be filled by FEA)

## 1.3 Execution

Test Setup

- Direction of Acceleration                 Forward direction

- Test Setup                                         Description how to mount the fixture + Bpilar + foot rest on the sled (example of LH seat):

- Camera Setup Overview                   1 - Mid Left 90° | 2 - Mid Right 90° |

- Camera Setup Detail                         3 -Front Left 45° | 4 -Rear Right 45°

Test Position

- Adjust the tracks to 110mm forward form the fully rearward position

- For locking the tracks proceed to the below steps:

  - 1 - Fully rearward to put the screw IN

  - 2 - Fully forward to put the screw IN

  - 3 - Driving minimum 4 times between the end stops fully forward and fully rearward

  - 4 - Sliding the to the test position until locking

  - 5 - Shaking seat on tracks

  - 6 - Checking locking pin and mark it with white marker

- Cushion Height Position                    N/A

- Headrest Height Position		1 - Adjusted to the height of the head => Z-backset closest to 0mm

2 - Adjust the headrest in X to the fully rearward position

- Track Position			110mm forward from the fully rearward position

For locking the tracks proceed to the below steps:

1 - Fully rearward to put the screw IN

2 - Fully forward to put the screw IN

3 - Driving minimum 4 times between the end stops fully forward and fully rearward

4 - Sliding the to the test position until locking

5 - Shaking seat on tracks

6 - Checking locking pin and mark it with white marker

- Backrest Angle			Backrest to be adjusted in design position=> angle from recliner					pivot point to headrest bezel exit centre point = 14.5°

- Headrest Height Position		1 - Adjusted to the height of the head => Z-backset closest to 0mm

2 - Adjust the headrest in X to the fully rearward position

- Fastening Parts			Original Renault / Nissan screw and reducing head diameter

to 17.8mm as per original screw drawing |  | Torque = 35Nm for

each of the 4 screws |  | Torque = 35Nm for each of the 4 screws

|  | For belt buckle, use original screw + washer with a torque of 25Nm

- Test Procedure

- Preparations

  - 1- Mark the belt every 100mm from the D-loop to the					retractor and further

  - 2 - Mark all the mechanisms with a white marker

  - 3 - Fix markers like described under "Additional Information Video Analyze" and measure Y-distance to the camera

  - 4 - At the buckle area, mark 15 points with interval of 10mm each on the cushion side-member and measure this points with faro arm

- Pre Test Measurements

  - 1 - Measure with faro arm the buckle fixing point coordinates X / Y / Z

  - 2 - At he buckle area, mark 15 points with interval of 10mm each on the cushion side-member and measure this points with faro arm the X / Y / Z coordinates as described in the test preparation section

  - 3 - Measure with faro arm the X-Y-Z coordinates at the IB and OB FSC side-member nose area.

  - 4 - Measure the unlocking force at the buckle

- Post Test Measurements

  - 1 - Measure with faro arm the buckle fixing point coordinates X / Y / Z and calculate gap to pre-test

  - 2 - At he buckle area, mark 15 points with interval of 10mm each on the cushion side-member and measure this points with faro arm the X / Y / Z coordinates as described in the test preparation section and calculate gap to pre-test

  - 3 - Measure with faro arm the X-Y-Z coordinates at the IB and OB FSC side-member nose area and calculate gap to pre-test

  - 4 - Measure the unlocking force at the buckle and calculate gap to pre-test

  - 5 - Take picture of the lap belt if still in contact with the Iliac bone area for submarining rating

Evaluation Method

Video Analysis

1 - Pelvis displacement in X and Z => Refer to "Sketch / Picture Test Setup.

2 - Headrest centre line x-tracking without untrimming headrest.

Additional Information Video Analysis

- Add the following targets on the dummy and measure the distance to the camera : 8 targets totally to be recorded.

=> Cog Head IB / OB

=> Shoulder IB / OB

=> Knee IB / OB

=> Ankle IB / OB

=> H-point IB / OB

=> Heel point IB / OB

- Add the following targets on the seat and measure the distance to the camera : (17 Targets totally to be recorded.)

=> Headrest IB / OB / TOP

=> Headrest rod IB / OB

=> Top of the back frame IB / OB as per below instruction (untrim locally to fix it on the frame) :

=> Recliner pivot point IB / OB (remove the OB side valence if necessary)

=> Cushion side-member nose area as per below description:

=> Buckle fixation point

=> Buckle

Calculating the pelvis H-point by tracking the femur:

Table 2 Calculation of the Pelvis H-point

Initial position of the thigh model			Maximum forward position of the thigh model

Before Test:

- Adjust the dummy in order to fullfill Hpoint and pelvis angle corridor.

- Attach on the femur plate the targets P and F in order that Hpoint + target.

- P + Target F are on one line.

- Measure the distance A between the H point and P femur pattern and angle alpha by recording with faro arm the Hpoint and target P and F coordinates.

After test:

By using tracking software;

- Measure on the first frame of the movie the angle Alpha formed by the straight line passing through the center of two femur patterns P and F and horizontal.

- Measure at the maximum advanced position of the pelvis the angle Beta formed by the straight line passing through the center of the two femur patterns P and F and horizontal.

- Measuring ΔXp at advanced X position of the point P.

- Measuring ΔZp at advanced Z position of the point P.

Calculate the coordinates of H advanced such as:

Hx= A cos α + ΔXP – A cos β

Hz= A sin α + ΔZP – A sin β

## 1.4 Setup

- Front seat mounted on rigid fixture with standard screws

- Fixture mounted on sled with standard screws

- Seat occupied with 50% HIII instrumented dummy

- Original belt system according to vehicle coordinates

- Footrest mounted on sled according to vehicle coordinates with standard screws

Table 3 Pre Test Adjustments

| Seat Adjustments | LH/ pos. 01 |
| --- | --- |
| Track position | Fully Rearward |
| Cushion angle | N/A |
| Backrest angle | 14.5° |
| Headrest height | Z-backset closes to 0 |