# 1. Test Execution and Setup

## 1.1 Test Description

Pulse ID: J050271

## 1.2 Test / Part Criteria

General Data

- Test Regulation		Test performed according to the Stellantis test procedure B32-3210

- Sled Pulse [Number] 	Biltir: 26002Y018E008

- Test Object                          Test applicable on K0 complete front seat

- Test Fixture                         K0 front seat crash fixture for metal validation (simulating podium as rigid)

K0 footrest

K0 B-pilar

- Type of Dummy		HIII 50% M

- Dummy Setup Information

| Parameter | Coord. | Target | Actual |
| --- | --- | --- | --- |
| H-Point [mm] | X | 1163,97 < Hx < 1189,97 | 1182.29 |
|  | Y | +/-380 | -416.65 |
|  | Z | 614,38 < Hz < 640,38 | 631.32 |
| Heel Point at the footrest [mm] | X | 566.5 | 490 |
|  | Y | N.A. | N/A |
|  | Z | 275 | 276.3 |
| Feet angle [°] at the footrest | / | 40.0 | 40.1 |
| Pelvis Angle [°] | / | 22.5 ± 2.5 | 24.10 |
| Head Angle [°] | / | 0 ± 2.5 around Y and X-axis | 1.55/0.70 |
| Head COG [mm] | X | To record X-backset for info | Not necessary for this test |
|  | Z | To record Z-backset for info | Not necessary for this test |

| - Belt System | 1 - Original belt buckle for K0 project NAFTA variant 2 - Original PLP for K0 project NAFTA variant 3 - Original belt for K0 project with 600daN NAFTA variant Belt elongation = (Input from FEA required) |
| --- | --- |

- Belt Coordinates D-Loop

D-loop fixing point:

X = 1480.7 mm

Y = + 721.1 mm

Z = 1276.3 mm

Retractor fixing point:

X = 1413.09 mm

Y = + 752.9 mm

Z = 388.7 mm

- Belt Coordinates Lower Outboard

PLP fixing point on Bpilar:

X = 1425.2 mm

Y = + 700.3 mm

Z = 251.7 mm

- Belt Force Sensor                              1 - Diagonal belt sensor => Distance to the D-ring = 200mm

2 -  Lap belt sensor outboard => Minimum distance to the outboard low fixation = 150mm

- Firing Time Pretensioner [ms]	PLP = 20 ms

The PLP device shall be blocked (integrated in the Bpilar design)

- Firing Time Retractor [ms]		Retractor = 10 ms

- Additional information		Pull in force = (input to be filled by FEA)

pretensioner 			Pull in displacement = (input to be filled by FEA)