## 1.3 Execution

Test Setup

- Direction of Acceleration                 Forward direction

- Test Setup                                         Description how to mount the fixture + Bpilar + foot rest on the sled (example of LH seat):

- Camera Setup Overview                   1 - Mid Left 90° | 2 - Mid Right 90° |

- Camera Setup Detail                         3 -Front Left 45° | 4 -Rear Right 45°

Test Position

- Adjust the tracks to 110mm forward form the fully rearward position

- For locking the tracks proceed to the below steps:

  - 1 - Fully rearward to put the screw IN

  - 2 - Fully forward to put the screw IN

  - 3 - Driving minimum 4 times between the end stops fully forward and fully rearward

  - 4 - Sliding the to the test position until locking

  - 5 - Shaking seat on tracks

  - 6 - Checking locking pin and mark it with white marker

- Cushion Height Position                    N/A

- Headrest Height Position		1 - Adjusted to the height of the head => Z-backset closest to 0mm

2 - Adjust the headrest in X to the fully rearward position

- Track Position			110mm forward from the fully rearward position

For locking the tracks proceed to the below steps:

1 - Fully rearward to put the screw IN

2 - Fully forward to put the screw IN

3 - Driving minimum 4 times between the end stops fully forward and fully rearward

4 - Sliding the to the test position until locking

5 - Shaking seat on tracks

6 - Checking locking pin and mark it with white marker

- Backrest Angle			Backrest to be adjusted in design position=> angle from recliner					pivot point to headrest bezel exit centre point = 14.5°

- Headrest Height Position		1 - Adjusted to the height of the head => Z-backset closest to 0mm

2 - Adjust the headrest in X to the fully rearward position

- Fastening Parts			Original Renault / Nissan screw and reducing head diameter

to 17.8mm as per original screw drawing |  | Torque = 35Nm for

each of the 4 screws |  | Torque = 35Nm for each of the 4 screws

|  | For belt buckle, use original screw + washer with a torque of 25Nm