Evaluation Method

Video Analysis

1 - Pelvis displacement in X and Z => Refer to "Sketch / Picture Test Setup.

2 - Headrest centre line x-tracking without untrimming headrest.

Additional Information Video Analysis

- Add the following targets on the dummy and measure the distance to the camera : 8 targets totally to be recorded.

=> Cog Head IB / OB

=> Shoulder IB / OB

=> Knee IB / OB

=> Ankle IB / OB

=> H-point IB / OB

=> Heel point IB / OB

- Add the following targets on the seat and measure the distance to the camera : (17 Targets totally to be recorded.)

=> Headrest IB / OB / TOP

=> Headrest rod IB / OB

=> Top of the back frame IB / OB as per below instruction (untrim locally to fix it on the frame) :

=> Recliner pivot point IB / OB (remove the OB side valence if necessary)

=> Cushion side-member nose area as per below description:

=> Buckle fixation point

=> Buckle

Calculating the pelvis H-point by tracking the femur:

Table 2 Calculation of the Pelvis H-point

Initial position of the thigh model			Maximum forward position of the thigh model

Before Test:

- Adjust the dummy in order to fullfill Hpoint and pelvis angle corridor.

- Attach on the femur plate the targets P and F in order that Hpoint + target.

- P + Target F are on one line.

- Measure the distance A between the H point and P femur pattern and angle alpha by recording with faro arm the Hpoint and target P and F coordinates.

After test:

By using tracking software;

- Measure on the first frame of the movie the angle Alpha formed by the straight line passing through the center of two femur patterns P and F and horizontal.

- Measure at the maximum advanced position of the pelvis the angle Beta formed by the straight line passing through the center of the two femur patterns P and F and horizontal.

- Measuring ΔXp at advanced X position of the point P.

- Measuring ΔZp at advanced Z position of the point P.

Calculate the coordinates of H advanced such as:

Hx= A cos α + ΔXP – A cos β

Hz= A sin α + ΔZP – A sin β

## 1.4 Setup

- Front seat mounted on rigid fixture with standard screws

- Fixture mounted on sled with standard screws

- Seat occupied with 50% HIII instrumented dummy

- Original belt system according to vehicle coordinates

- Footrest mounted on sled according to vehicle coordinates with standard screws

Table 3 Pre Test Adjustments

| Seat Adjustments | LH/ pos. 01 |
| --- | --- |
| Track position | Fully Rearward |
| Cushion angle | N/A |
| Backrest angle | 14.5° |
| Headrest height | Z-backset closes to 0 |